OS Requirements for installing Oracle 19c Single on IBM-AIX 7.x

| Requirements for Oracle Database (Single ASM) |  |
| --- | --- |
| OS version | AIX 7.x |
| Oracle DB Version / Single | 19c / Single + ASM |
| OS Requirements |  |
| AIX OS | AIX 7.x Technology Level 0 Service Pack 1 (7300-02-01-2148) or later, 64-bit kernel |
| System Filesets | bos.adt.base bos.adt.lib bos.adt.libm bos.perf.libperfstat bos.perf.perfstat bos.perf.proctools xlC.aix61.rte.13.1.2.0 or later xlC.rte.13.1.2.0 or later xlfrte - The following filesets are required for Oracle Real Application Clusters (Oracle RAC) Installations only: rsct.basic.rte rsct.compat.clients.rte - If you are using the minimum operating system TL level for AIX 7.3 listed above, then install the following AIX APAR fix: APAR IJ38518 |
| Kernel Settings | Ensure that the lower range is set to at least 9000 or higher, to avoid Well Known ports, and to avoid ports in the Registered Ports range commonly used by Oracle and other server ports. Set the port range high enough to avoid reserved ports for any applications you may intend to use. If the lower value of the range you have is greater than 9000, and the range is large enough for your anticipated workload, then you can ignore OUI warnings regarding the ephemeral port range tcp_ephemeral_low = 32768 tcp_ephemeral_high = 65500 udp_ephemeral_low = 32768 udp_ephemeral_high = 65500 |
| Shell limits | Soft File Descriptors at least 1024 Hard File Descriptors at least 65536 Number of processes (Soft) at least 2047 Number of processes (Hard) at least 16384 Soft STACK size at least 10240 KB Hard STACK size at least 10240 KB; at most 32768 KB Soft FILE size unlimited Soft CPU time unlimited Note:This is the default value. Soft DATA segment unlimited Soft Real Memory size unlimited |
| Swap memory | - 서버의 물리 메모리 기준으로 8G~16G 사이는 같은 크기로 Swap 메모리 설정 - 물리 메모리가 16G 이상이면 Swap 메모리는 16G로 설정 |
| Enable I/O Completion Ports | IOCP 상태를 Available로 변경 $ lsdev ¦ grep iocp iocp0 Available I/O Completion Ports |
| Network |  |
| Public IP Address | TCP/IP 지원 DNS 및 /etc/hosts에 등록 |
| HOSTNAME 설정 | 서버 Hostname에는 특수문자 및 _ 등을 사용하지 않고 설정 |